Robart SDK Local HTTP API

Complete Endpoint Reference

Scope: Rowenta / Tefal RobEye robot vacuums using the Robart SDK local HTTP API (port 8080). Consolidated from all live-device captures, APK (Rowenta Robots v9.5.1-RC1) DEX/asset extraction, the RobEye web UI HTML, and the rowenta_roboeye integration codebase.

Confidence legend:  LIVE = confirmed from live device  ·  APK = confirmed in app bytecode/assets  ·  HTML = confirmed in robot web UI  ·  INFER = inferred, needs live test  ·  DEAD = exists but unusable on this firmware

Transport: All endpoints are HTTP GET (even commands). Base: http://<robot_ip>:8080.  Coordinate system (LIVE): 1 API unit = 2 mm = 0.2 cm. Y increases upward (flip for SVG). /get/rob_pose returns heading in degrees; legacy debug/* endpoints return raw (÷ 65536/360).

# 1.  Platform & Model Taxonomy

Model detection comes from parseRobotType (APK bytecode). The robot's uniqueId prefix is the primary discriminator, then sub-type by name.

| Platform | Models | uniqueId | Setup | Auth | Mopping | SDK / 8080 |
| --- | --- | --- | --- | --- | --- | --- |
| LEGACY | X-Plorer Serie 120, S220, S240 | not aicu- | WiFi AP hotspot | None (HTTP always open) | No | Yes |
| AICU (modern) | Serie 130, 140, 375 | starts aicu- | BLE | 8-char password | varies by sub-type | Yes |
| └ HELIOS | name contains Helios | aicu- | BLE | 8-char | Yes (wet mop) | Yes |
| └ L6 | name == Agon or contains HY100 | aicu- | BLE | 8-char | Yes (wet mop) | Yes |
| └ L7 | name == Agonoa | aicu- | BLE | 8-char | Yes (wet mop) | Yes |
| └ RC100 | name contains RC100 | aicu- | BLE | 8-char | No | Yes |
| └ C5 | name contains Chronos20 | aicu- | BLE | 8-char | No | Yes |
| Tuya | Serie 50–80, S85+ | n/a | Tuya cloud | Tuya | n/a | No — OUT OF SCOPE |

- isModern() is true for both LEGACY and AICU (UNKNOWN ≠ LEGACY). Only Tuya models are excluded.

- The HTTP API is identical across LEGACY and AICU except for (a) auth and (b) mopping endpoints.

- Cross-reference: the official ROMY HA integration (homeassistant/components/romy, mDNS _romy._tcp.local) and the openHAB ROMY binding both drive the same Robart SDK get/set scheme.

# 2.  Authentication Model

| Platform | Behaviour |
| --- | --- |
| LEGACY (120/S220/S240) | LIVE HTTP permanently open. No password ever set. lock_http/unlock_http exist in SDK but irrelevant. |
| AICU (130/140/375/Helios/L6/L7) | APK 8-char password (ROBOT_PASSWORD_SIZE = 8) from QR sticker on robot during BLE onboarding. |

AICU auth flow (APK):

- lock_http is NOT called by default → HTTP stays open even on AICU.

- If the native app enabled lock: every request needs Authorization: Basic base64(robot_unique_id:password).

- GET /set/unlock_http?pass=<8-char> re-opens HTTP.

- Integration: optional http_password config field (blank = no auth). 401 → ConfigEntryAuthFailed.

# 3.  GET Endpoints — Robot State & Info

| Endpoint | Params | Confirmed response (key fields) | Poll | Models | Conf |
| --- | --- | --- | --- | --- | --- |
| /get/status | — | battery %, mode, charging, fan speed raw. AICU wet adds active_pump_volume. | 2–15 s | all | LIVE |
| /get/robot_id | — | serial_number → device unique_id source | 3600 s | all | LIVE |
| /get/wifi_status | — | ssid, rssi | 3600 s | all | LIVE |
| /get/protocol_version | — | version firmware string | 3600 s | all | LIVE |
| /get/robot_flags | — | feature/state flags. AICU wet adds WATER_TANK_INSERTED, WATER_TANK_EMPTY, STUCK_WATER_PUMP, MISSING_WATER_PUMP, STUCK_WET_PAD_AGITATOR | 15 s | all | LIVE |
| /get/sensor_status | — | cliff / bumper / LiDAR health | diag | all | LIVE |
| /get/sensor_values | — | gyro / odometry + GPIO brush-stuck flags | 500 ms / 10 s cached | all | LIVE |
| /get/sensor_commands | — | alternate brush-stuck source (higher-level codes) | diag | all | LIVE |
| /get/product_feature_set | — | feature flags → detect HELIOS/L6/L7 wet support | setup | all | APK |
| /get/safety_mcu_firmware_version | — | secondary firmware version | setup | all | APK |
| /get/statistics | — | total distance/time/area/run count | 600 s | all | LIVE |
| /get/permanent_statistics | — | lifetime stats (e.g. total_distance_driven) | 600 s | all | LIVE |
| /get/cleaning_parameter_set | — | active fan/strategy params. AICU wet adds user_pump_volume | — | all | LIVE |
| /get/command_result | — | {"commands":[{"cmd_id":154,"status":"executing","error_code":0}]} — array, not single object | 3–5 s | all | LIVE |

/get/status mode strings: ready, cleaning, go_home. Charging: charging etc.

Recharge-and-continue: mode=cleaning + charging=charging persists for the whole dock+charge cycle (~100 min observed); the same cmd_id stays "executing" throughout.

# 4.  GET Endpoints — Maps & Areas

| Endpoint | Params | Confirmed response (key fields) | Poll | Conf |
| --- | --- | --- | --- | --- |
| /get/maps | — | permanent_flag is STRING "true". map_meta_data = user name (strip whitespace; empty → "Map N" by 1-based position). last_cleaned.year==2001 = never-cleaned sentinel. | 600 s | LIVE |
| /get/map_status | — | active map metadata / map_id confirmation | — | LIVE |
| /get/areas | map_id | Top-level (NOT in area_meta_data): cleaning_parameter_set, strategy_mode (STRING "normal"/"deep" only). area_state: inactive(unnamed)/clean. area_type: to_be_cleaned(spot/avoid). room_type: corridor/kitchen/sleeping/living/none. Named: area_meta_data = JSON with name. AICU wet adds pump_volume. | 300 s | LIVE |
| /get/schedule | — | see §7 | 300 s | LIVE |
| /get/n_n_polygons | map_id | named room boundary polygons (static floor plan) | 5 s SLAM | LIVE |
| /get/seen_polygon | map_id | visited/cleaned area polygon (segments x1/y1/x2/y2). Persists after dock. | 5 s | LIVE |
| /get/cleaning_grid_map | map_id | occupancy grid. lower_left_x/y API units, resolution=40 units = 8 cm/cell. map_id=3 persists last run after dock; map_id=41 is live-session variant. | 5 s | LIVE |
| /get/feature_map | map_id | walls (lines x1/y1/x2/y2) + dock position via docking_pose (heading raw; handle valid:'false'). | 600 s | LIVE |
| /get/tile_map | map_id | tile outline (outline x/y); also lists docking position | 600 s | LIVE |
| /get/topo_map | map_id | topological map graph | 600 s | LIVE |
| /get/points_of_interest | map_id | {"points_of_interest":[{}]} — empty on Serie 120; do not use as dock source | — | DEAD |

Dock position source: use /get/feature_map → docking_pose (NOT points_of_interest, which is empty on Serie 120).

# 5.  GET Endpoints — Position / Localization

| Endpoint | Params | Notes | Conf |
| --- | --- | --- | --- |
| /get/rob_pose | — | Authoritative position. {"x1":-2,"y1":-3,"heading":157,"valid":true,"is_tentative":false,"timestamp":958459}. heading already degrees. valid:false=no fix. Works in all states. Replaces all debug/* localization endpoints. | LIVE |
| /get/live_parameters | — | {x, y, heading} + area_cleaned, cleaning_time. Removed from all polling in favour of rob_pose. | LIVE |
| /debug/localization | — | idle: localization_algo_input[] → use "global" entry. Superseded by rob_pose. | LIVE |
| /debug/relocalization | — | cleaning on saved map: use last "continuous" entry. Superseded. | LIVE |
| /debug/exploration | — | new map: exploration_points[] → max ts. Superseded. | LIVE |
| /debug/smsc | — | low-level diagnostic | LIVE |

# 6.  GET Endpoints — Logs & History

| Endpoint | Params | Confirmed response (key fields) | Conf |
| --- | --- | --- | --- |
| /get/event_log | last_id | {"robot_events":[{id,type,type_id,timestamp,current_status,map_id,area_id,source_type,source_id,hierarchy,info}]}. last_id=N → events with id > N. source_type: user=command, operation_unit=hardware. | LIVE |
| /get/ui_cmd_log | — | [{id,cmd,rtc{...},params,source}]. source=0 = from app/UI. cmd names below. | LIVE |
| /get/task_history | — | past cleaning task records | LIVE |
| /get/critical_logs | — | critical errors → HA Repairs panel | LIVE |
| /get/bug_report | — | diagnostic bundle | LIVE |

Confirmed event_log type_id values:

2000 battery_low   2010 robot_lifted   2011 robot_setback   2030 dustbin_missing   2031 dustbin_inserted
1011 clean_area succeeded     1012 clean_area interrupted     1033 go_home failed
1040 recharge_and_continue started     1042 recharge_and_continue interrupted
1111 clean_map_areas succeeded     1170 redocking started     1172 redocking interrupted
1200 skipped (info=5 = insufficient battery)     hierarchy=3 = go_home sub-action inside recharge_and_continue

ui_cmd_log cmd names: ui_cmd_modify_area, ui_cmd_propose_nogo_areas, ui_suction_control_mode, ui_cmd_clean_map, ui_cmd_clean_start_or_continue, ui_cmd_go_home.  modify_area params show modified={...,scm:N} where scm = cleaning_parameter_set.

# 7.  Schedules

| Endpoint | Params | Notes | Conf |
| --- | --- | --- | --- |
| /get/schedule | — | Returns ALL (enabled+disabled). Entry fields below. | LIVE |
| /set/modify_scheduled_task | task_id, enabled (0/1) | Toggle enable/disable → {"cmd_id":N}. Only these two params required. Must bypass the command queue. | LIVE |
| /set/add_scheduled_task | (full task) | Add new schedule. AICU wet adds pump_volume. | APK |

/get/schedule entry: task_id, enabled(int 0/1), time{days_of_week[1=Mon..7=Sun], hour, min, sec}, task{map_id, cleaning_parameter_set(0=per-room default,1=Eco,2=Normal,3=High,4=Silent), cleaning_mode(1=all rooms,2=specific), parameters[area_ids]}.

parameter1/parameter2 are string duplicates of parameters[0]/[1] — ignore them, use parameters[].

# 8.  SET Endpoints — Commands

| Endpoint | Params | Notes | Models | Conf |
| --- | --- | --- | --- | --- |
| /set/clean_all | cleaning_parameter_set(1–4), cleaning_strategy_mode(numeric, 3=Deep), method | Clean whole map | all | LIVE |
| /set/clean_map | map_id, area_ids(comma-list), cleaning_parameter_set, cleaning_strategy_mode, AICU: pump_volume | Clean specific rooms. Multi-room = comma-separated ids in one call → one cmd_id. Also the only way to switch active map (?map_id=X). | all | LIVE |
| /set/clean_start_or_continue | — | Resume after user-pause → brand new cmd_id (original → aborted). Use this, NOT clean_continue. | all | LIVE |
| /set/go_home | — | Return to dock | all | LIVE |
| /set/stop | — | Stop. Transitions in-progress cmd_id → "aborted" (not "done"). | all | LIVE |
| /set/switch_cleaning_parameter_set | cleaning_parameter_set(1–4) | Set fan speed. Must bypass queue (settings write). | all | LIVE |

Strategy / fan-speed numbering — these DIFFER across endpoints:

- clean_* cleaning_strategy_mode (numeric): 1=Normal, 2=Walls/Corners, 3=Deep → STRATEGY_DEEP="3".

- /set/modify_area strategy_mode (STRING): only "normal" / "deep" accepted.

- /get/areas strategy_mode (STRING): "normal" / "deep".

- modify_area fan speed: 1=Silent, 2=Eco, 3=High, 4=Super Silent.

- Suction levels elsewhere: 0=Default, 1=Normal, 2=Silent, 3=Intensive, 4=Super Silent.

- Schedule cleaning_parameter_set: 0=per-room default, 1=Eco, 2=Normal, 3=High, 4=Silent.

method parameter (HTML/INFER): RobEye web UI exposes none/dry/wet for clean_all/clean_map. String vs numeric form unconfirmed — live-test both &method=wet and &method=2. Irrelevant on dry-only LEGACY units.

# 9.  SET Endpoints — Area / Map Editing

All flat query params, no JSON body. No save_map call needed after.

| Endpoint | Params | Notes | Conf |
| --- | --- | --- | --- |
| /set/modify_area | map_id, area_id, cleaning_parameter_set, strategy_mode("normal"/"deep") | Per-area settings write. Persists across HA restarts. Must bypass queue. | LIVE |
| /set/add_area | x1,y1,x2,y2,x3,y3,x4,y4, area_meta_data='{"name":""}', area_state, cleaning_parameter_set, area_type | Blocking zone: area_state=blocking, cps=0, area_type=to_be_cleaned. Spot: area_state=clean, cps=1. | APK |
| /set/merge_areas | area_id1, area_id2 | NOTE: area_id1/2, not area_id_1/_2 | APK |
| /set/split_area | area_id, x1,y1,x2,y2 | Two boundary intersection points (NOT points=JSON) | APK |

# 10.  AICU-Only Mopping Endpoints

Only for HELIOS / L6 / L7 (hasWetSupport()==true). LEGACY and RC100/C5 have no mopping hardware — skip entirely.

| Endpoint | Params | Notes | Conf |
| --- | --- | --- | --- |
| /get/pump_volume_settings | — | {"mode":"low¦medium¦high¦auto¦none"} | APK |
| /set/pump_volume_settings | mode(low/medium/high/auto/none) | Set water flow | APK |
| /set/live_parameters | do_wet_clean(true/false) | On/off wet-clean toggle during a run | APK |

Wet fields appended to existing payloads: pump_volume (clean_map, add_scheduled_task, /get/areas), user_pump_volume (/get/cleaning_parameter_set), active_pump_volume (/get/status). Gate registration of mopping entities on hasWetSupport() at setup.

# 11.  Auth / Lock Endpoints

| Endpoint | Params | Notes | Conf |
| --- | --- | --- | --- |
| /set/lock_http | — | Require Basic auth on all requests. Not called by default. | APK |
| /set/unlock_http | pass(8-char) | Re-open HTTP | APK |

# 12.  Dead / Unsupported Endpoints (Serie 120 firmware)

| Endpoint | Result | Action |
| --- | --- | --- |
| /get/rooms | unknown_request | Don't call — use /get/areas |
| /get/configurable_parameters | 400 Bad Request | Don't call |
| /get/points_of_interest | {"points_of_interest":[{}]} empty | Don't use for dock |
| /set/use_map | error 101 | Switch maps via /set/clean_map?map_id=X instead |
| /set/clean_continue | error 106 command_deprecated (does nothing) | Use /set/clean_start_or_continue |

# 13.  APK-Confirmed Endpoint List (v9.5.1-RC1)

GET: rob_pose, rooms, product_feature_set, safety_mcu_firmware_version, event_log?last_id=0.

SET: clean_all, clean_map, go_home, stop, switch_cleaning_parameter_set, modify_scheduled_task.

# 14.  Architectural Rules (cross-cutting)

- Queue bypass (settings writes): modify_area, switch_cleaning_parameter_set, modify_scheduled_task must always bypass the asyncio.Queue — they are settings writes, not cleaning operations.

- cmd_id from /set/ responses is the authoritative tracking handle; _wait_for_cmd treats both done and aborted as terminal; detects recharge-and-continue and extends timeout to 3 h (poll every 30 s).

- Polling tiers: 500 ms base (sensor_values), 2 s status, 5 s SLAM (polygons/grid/rob_pose), 10 s brush-stuck (cached), 15 s idle, 300 s areas/schedule, 600 s map geometry/statistics, 3600 s robot info.

- Map switching is manual (select.active_map); no use_map endpoint exists.

# 15.  Deep-Clean Feature & Entity Naming (open item)

Per-room deep clean as a SwitchEntity:

- Unique-id scheme: [device]_map{N}_room_{id}_[property], e.g. rowenta_xplorer_120_map3_room_10_deep_clean (map prefix required for multi-floor).

- Write path: /set/modify_area?...&strategy_mode=deep (string form) — bypasses queue.

- clean_map/clean_all use numeric cleaning_strategy_mode=3 for a one-shot deep run.

- HA is source of truth for per-room strategy after setup.

UI references: Tasshack/dreame-vacuum (HA UI patterns) and PiotrMachowski/lovelace-xiaomi-vacuum-map-card (interactive room-selection map card).

Sources: live device captures (X-Plorer Serie 120, fw SER120-1.1.0-release:3.11.2872, maps 3/45), Rowenta Robots v9.5.1-RC1 APK DEX/asset extraction, RobEye web UI HTML, rowenta_roboeye codebase, and ApolloLogs endpoint enumeration. Cross-reference: home-assistant romy component, openHAB ROMY binding.